AcoustiScan

Step-by-step guide for the team

Set up your laptop, wire 4 ESP boards, connect them to Wi-Fi and see them in the dashboard. No experience needed.

How to use this guide - Do the Parts in order, 1 to 9. Do not skip ahead. - A grey box with typewriter text is something to copy and paste. Most of them are messages for the AI helper (called prompts). - You should see tells you what success looks like. If it does not work tells you what to try. - The AI helper does the typing for you. You do the physical things: plug in cables, connect the wires, click Allow. - If you get stuck, go to Part 8. Do not guess: copy what the AI wrote and ask a teammate.

| Part | What you do | When |
| --- | --- | --- |
| 1 | Understand what you need and what the words mean | Once |
| 2 | Install the programs on your laptop | Once |
| 3 | Prepare the Wi-Fi and connect the laptop to it | Once |
| 4 | Let the AI prepare the laptop | Once |
| 5 | Connect the microphone to a board with 5 wires | For each board |
| 6 | Set up each ESP with the AI (ESP-1 to ESP-4) | For each board |
| 7 | Use the dashboard | Every day |
| 8 | When something goes wrong | When needed |
| 9 | Checklist and the same steps without AI | Reference |

# Part 1. What you need and what the words mean

## 1.1 What the system does

Each ESP board carries a small microphone. The boards join your Wi-Fi. On your laptop you open a web page called the dashboard. You click a board, choose how many seconds to record, and the board sends the sound to the laptop. The dashboard then shows the sound as graphs and saves it as a file.

The boards do not need internet. Only the Wi-Fi between the laptop and the boards matters.

ESP-1 + mic ---+ ESP-2 + mic ---+---- same Wi-Fi ---- Laptop with the dashboard ESP-3 + mic ---+ http://127.0.0.1:8080 ESP-4 + mic ---+

## 1.2 What you need

| Item | How many | Notes |
| --- | --- | --- |
| ESP32-S3 board (ESP32-S3-WROOM-1 N16R8, DevKitC-1 style) | 4 | It has two USB-C sockets. One is printed COM, the other USB. We always use COM. |
| Microphone module Knowles SPH0641LU4H-1 | 4 | One per board. Its pins are named 3.3, GND, CLK, DATA, SEL. |
| Jumper wires | 20 | 5 wires per microphone. |
| USB-C data cable | 1 or more | A charge-only cable does not work. If the laptop does not see the board, change the cable first. |
| Phone chargers or power banks | 4 | To power the boards after the setup. |
| Windows laptop | 1 | Windows 10 or 11. |
| Wi-Fi: phone hotspot or home router | 1 | 2.4 GHz is required. Internet is not needed. |
| An AI helper that can run commands in VS Code | 1 account | We use Claude Code (Part 2). |

## 1.3 Words you will see

| Word | Meaning |
| --- | --- |
| ESP, board, node | One ESP32 board with its microphone. We name them ESP-1, ESP-2, ESP-3, ESP-4. |
| Dashboard | The web page on the laptop where you record and see the results. |
| VS Code | A free program. You open the project folder in it and talk to the AI in it. |
| PlatformIO | An add-on for VS Code that puts programs onto the boards. |
| Flash | Copy the program onto the board. |
| Provision | Give a board its name (for example ESP-1) and the Wi-Fi name and password. Done once, with a USB cable. |
| COM port | The name Windows gives to the USB socket the board is plugged into (for example COM8). |
| Prompt | The message you paste into the AI chat. |
| Hotspot | Wi-Fi shared by your phone. |
| Firewall | Windows protection. It can block the boards until you click Allow. |

## 1.4 Where the project is today

Status Done and tested: a board joins Wi-Fi, the microphone records, and the software was built and tested with simulated boards. Not tested yet: the whole chain with real boards. Setting up ESP-1 (Part 6) is that first real test. If something fails, that is normal for a first test: use Part 8. Planned later: record from several boards at the same time, keep the boards' clocks in step, find where a sound comes from, use a dedicated router, go from 4 to 8 boards.

# Part 2. Install the programs on your laptop

This takes 30 to 60 minutes, mostly waiting for downloads. You need internet for this part.

### Step 2.1  Install VS Code

- Open code.visualstudio.com in your browser and download VS Code for Windows.

- Run the installer. Click Next and accept the default options.

### Step 2.2  Install Python 3.14

- Open python.org/downloads and download Python 3.14 for Windows. Run the installer and accept the default options.

- When it is finished, open PowerShell (press the Windows key, type PowerShell, press Enter). Type py -3.14 --version and press Enter.

You should see A line like Python 3.14.x.

If it does not work "py is not recognized": run the Python installer again and keep the option for the py launcher ticked. Then open a new PowerShell window.

### Step 2.3  Get the project folder

A teammate sends you a file named acoustiscan.zip. Right-click it, choose Extract All, and extract it to C:\AcoustiScan. Inside that folder you must see a file named platformio.ini.

For the person who shares the folder The best way is a private GitHub repository: the project is already set up so that passwords and recordings are not uploaded. The other way is a zip. Run this in PowerShell on the PC that has the project. It leaves out the Wi-Fi password and the big temporary folders, and puts acoustiscan.zip on the Desktop:

$src = 'C:\gradproject 2' $tmp = Join-Path $env:TEMP 'acoustiscan_share' robocopy $src $tmp /E /XD .venv .pio captures __pycache__ /XF wifi_credentials.h /NFL /NDL /NJH /NJS Compress-Archive -Path "$tmp\*" -DestinationPath "$env:USERPROFILE\Desktop\acoustiscan.zip" -Force

Important Never send the folders .venv and .pio or the file include\wifi_credentials.h. The first two will not work on another PC, and the last one contains a real Wi-Fi password.

### Step 2.4  Open the folder in VS Code

- Start VS Code. Choose File > Open Folder... and select C:\AcoustiScan.

- Click Yes, I trust the authors.

- If VS Code offers to install recommended extensions, click Install. If not: click the Extensions icon in the left bar (four squares), search for PlatformIO IDE and click Install.

- Wait. PlatformIO installs itself in the background. This can take up to 10 minutes. If VS Code asks you to reload the window, do it.

You should see The PlatformIO ant-head icon in the left bar, and no more "installing" messages in the bottom bar.

### Step 2.5  Install the AI helper

- In VS Code, click the Extensions icon, search for Claude Code, click Install, and sign in with your account.

- Open the AI chat panel by following the extension's instructions. You will paste the prompts of this guide into its chat box.

- When the AI wants to run a command, it asks you first. Read it and click the button that allows it.

Another AI tool that can run commands in the VS Code terminal also works. The prompts in this guide were written for Claude Code.

# Part 3. Prepare the Wi-Fi

Most problems come from the Wi-Fi, so read the rules first.

The rules - The laptop and all four boards must use the same Wi-Fi network. - It must be 2.4 GHz. The boards cannot use 5 GHz. - A normal name and password only. University and hotel Wi-Fi with a login page will not work. - On a router, "client isolation" or "AP isolation" must be OFF. A phone hotspot normally allows this, but ESP-1 (Part 6) is the first real test of ours. - Internet is not needed. A hotspot with mobile data switched off works.

### Step 3.1  Create the hotspot (Samsung Galaxy S24 Ultra)

- Open Settings > Connections > Mobile Hotspot and Tethering > Mobile Hotspot.

- Tap Configure. Choose a Wi-Fi name and a password. Write both down: you need them in Part 4.

- Set Band to 2.4 GHz. Save.

- Switch the hotspot on. Keep it on during the whole setup. A phone can switch a hotspot off when nothing uses it.

Another phone: look for the same settings (hotspot name, password, band) and choose 2.4 GHz.

### Step 3.2  Connect the laptop

Click the Wi-Fi icon at the bottom right of the laptop screen and connect to your hotspot.

Mobile data Part 4 downloads several hundred MB. If your phone plan is small, do Part 4 on a normal Wi-Fi first and connect to the hotspot after it.

### Step 3.3  Windows firewall (you will do this once)

The first time the dashboard starts, Windows shows a Windows Security Alert saying that the firewall has blocked some features of Python. Tick both boxes, Private networks and Public networks, then click Allow access. If you tick only one, the boards will not appear. If you missed it, see Part 8.

# Part 4. Let the AI prepare the laptop

Now the AI installs everything the project needs and tests it. You only paste one prompt and do one small thing by hand.

- In VS Code, make sure the folder C:\AcoustiScan is open.

- Open the AI chat panel and start a new chat.

- Copy everything inside the grey box below, paste it into the chat and press Enter.

- Approve the commands when the AI asks. The first compile downloads a lot and can take several minutes.

PROMPT 0: prepare the laptop

I am a complete beginner. This VS Code window has the AcoustiScan project open. It is a system of ESP32-S3 boards with microphones plus a dashboard on this laptop. Please prepare this laptop so that I can put the program on the boards and run the dashboard. Explain each step in simple words, one or two lines at a time. Tell me clearly whenever I have to do something by hand. Do these steps in order. If one fails, stop and tell me what is wrong and what I should do: 1. Check that the PlatformIO tool exists at %USERPROFILE%\.platformio\penv\Scripts\pio.exe. If it does not, tell me to wait until the PlatformIO extension has finished installing (bottom of the VS Code window) and then ask you again. 2. Check that Python 3.14 is installed (py -3.14 --version). If it is not, tell me to install it from python.org and then ask you again. 3. Create the Python environment in the project folder (py -3.14 -m venv .venv) and install the packages from server\requirements.txt into it using .venv\Scripts\python.exe -m pip. 4. If include\wifi_credentials.h does not exist, create it by copying include\wifi_credentials.example.h. Then tell me to open that file myself, type my Wi-Fi name and my Wi-Fi password between the quotation marks, and save it. Do NOT ask me to paste the password into this chat, and never print the password anywhere. 5. Compile the board program to prove the tools work: pio run -e node. The first time it downloads a lot of files and can take several minutes. It must end with SUCCESS. 6. Test the dashboard without any real board: start tools\fake_node.py --id FAKE-1 and server\app.py --no-browser in the background with .venv\Scripts\python.exe, wait up to 15 seconds, check that http://127.0.0.1:8080/api/nodes lists FAKE-1 with online=true, then stop both programs again. Rules: do not change any source file or any GPIO pin number. Use only this project's .venv Python and the PlatformIO tool above. At the end give me a short summary: what passed, what failed, and what I still have to do by hand.

### The one thing you do by hand: your Wi-Fi name and password

The AI creates a file named wifi_credentials.h and asks you to fill it in. It does this so that the password never goes into the chat.

- In VS Code, in the file list on the left, open the folder include and double-click wifi_credentials.h.

- Replace PUT_MY_HOTSPOT_NAME_HERE with your Wi-Fi name and PUT_MY_PASSWORD_HERE with its password. Keep the quotation marks.

- Press Ctrl+S to save.

#define WIFI_SSID "MyHotspot" #define WIFI_PASSWORD "MyPassword123"

You should see At the end the AI says that the tools were found, the compile ended with SUCCESS, and the test board FAKE-1 was listed as online. Then Part 4 is finished and you never repeat it on this laptop.

If it does not work Read what the AI says is missing, do that one thing, and tell the AI "I did it, please continue from where you stopped". If the AI cannot fix it, go to Part 8.

# Part 5. Connect the microphone to a board

Important Do this with the board unplugged from USB. Every board is wired the same way. The program expects exactly these pins, so do not use other pins.

| Wire from the microphone pin | To this pin on the ESP board |
| --- | --- |
| 3.3 | 3V3 (3.3 volt). Never 5V. |
| GND | GND (any GND pin) |
| CLK | Pin 4 (also written GPIO4) |
| DATA | Pin 5 (also written GPIO5) |
| SEL | GND (do not leave it unconnected) |

- Put the board on a table with the USB sockets facing you.

- Connect the five wires from the table above.

- Check each wire a second time. A wire on the wrong pin is the most common reason for a dead microphone.

- Stick a label on the board: ESP-1, ESP-2, ESP-3 or ESP-4. Never give two boards the same label.

The two USB-C sockets: use the one printed COM. Plug in only one cable at a time.

# Part 6. Set up each ESP with the AI

You do the same routine for every board, with a different prompt for each. Do them in order: ESP-1 also tests the Wi-Fi, so it comes first.

## The routine (for every board)

- The microphone is wired and the board has its label (Part 5).

- The hotspot is on and the laptop is connected to it.

- Unplug every other board from the laptop. Plug only this board into the laptop, in the socket printed COM, with a data cable.

- In VS Code, open the AI panel and start a new chat, so that it forgets the previous board.

- Copy the prompt for this board (next pages) and paste it. Press Enter.

- Approve the commands. Wait until the AI gives its summary.

- Compare the summary with the table "What good looks like" below.

- Unplug the board from the laptop and plug it into a phone charger or power bank. After 10 to 20 seconds its LED turns green and it shows up in the dashboard again. Put it where you want it.

## What good looks like

| Check | Good | Not good: what it means |
| --- | --- | --- |
| COM port | One port found, for example COM8. | None found: charge-only cable, wrong socket, or missing driver. Try another cable and the COM socket. |
| Program on the board | Ends with SUCCESS. | "Port busy": close any other program that uses the board. "Cannot connect": hold the BOOT button, tap RST, release BOOT, and ask the AI to try again. |
| Naming (provision) | Ends with PROVISIONED and an IP address. | No IP after 40 seconds: hotspot is off, wrong Wi-Fi name or password, or the Wi-Fi is 5 GHz. |
| Dashboard | online true, mic alive. | Board missing: firewall or client isolation (Part 8). |
| Test recording | Result: OK. | "Mic flat": the microphone is not delivering data. Check the five wires (Part 5). |
| LED on the board | Green. | Blue: no Wi-Fi. Red: microphone problem. Yellow: the board has not been named yet. |

## Prompt for ESP-1 (first board, also tests the Wi-Fi)

Copy everything inside the grey box.

I am a beginner. Please set up AcoustiScan board ESP-1 (display name "Node 1") for me and show it in the dashboard. Explain each step in simple words, one or two lines at a time. Situation: this VS Code window has the AcoustiScan project open. Exactly one ESP32-S3 board is plugged into this laptop, in the USB socket marked COM. Its microphone is already wired. This is the FIRST board, so this also tests that my Wi-Fi network works. Do these steps in order. If one fails, stop and tell me what is wrong and what I should do: 1. Find the board's serial (COM) port with: pio device list (the PlatformIO tool is at %USERPROFILE%\.platformio\penv\Scripts\pio.exe). Use the port whose hardware ID contains 1A86:55D3, 10C4:EA60 or 303A. Ignore Bluetooth ports. If there is not exactly one such port, stop and ask me. 2. Without showing the password, check that include\wifi_credentials.h exists and no longer contains the text PUT_MY_. If it still does, tell me to open the file and type my Wi-Fi name and password myself (never ask me to paste the password here). Tell me which Wi-Fi name the file contains. 3. If the folder .venv does not exist, create it (py -3.14 -m venv .venv) and install server\requirements.txt into it. 4. Put the program on the board: pio run -e node -t upload --upload-port <port> 5. Give the board its name and Wi-Fi details: .venv\Scripts\python.exe tools\provision.py --port <port> --id ESP-1 --name "Node 1" It must end with the word PROVISIONED and an IP address. 6. If the dashboard is not already answering at http://127.0.0.1:8080/api/info, start it in the background: .venv\Scripts\python.exe server\app.py --no-browser 7. Check that ESP-1 is listed at http://127.0.0.1:8080/api/nodes with online=true and mic="alive" (wait up to 20 seconds). Then record 1 second as a test: send POST http://127.0.0.1:8080/api/capture with {"node_id":"ESP-1","seconds":1}, ask /api/jobs/<job_id> every second until it is done, then read stats.result at /api/captures/<capture_id>. The result should be "OK". 8. If step 5 printed an IP address but ESP-1 never shows up in step 7, run tools\udp_probe.py for 20 seconds and find out why. Usual reasons: the Windows firewall blocks python.exe on Public networks (UDP port 5005), the hotspot or router has "client isolation" switched on, or this laptop is on a different Wi-Fi network than the board. Explain what you found and ask me before changing any firewall setting. Rules: do not change any source file or any GPIO pin number. Use only this project's .venv Python and the PlatformIO tool. Never print the Wi-Fi password. At the end give me a short summary: COM port, IP address, signal strength (rssi), microphone status, and the test recording result. Then ask me to look at the LED on the board: green means good.

## Prompt for ESP-2

Copy everything inside the grey box.

I am a beginner. Please set up AcoustiScan board ESP-2 (display name "Node 2") for me and add it to the dashboard. Explain each step in simple words, one or two lines at a time. Situation: this VS Code window has the AcoustiScan project open. ESP-1 is already set up (it may be running somewhere else: do not touch it). Exactly one NEW board is plugged into this laptop, in the USB socket marked COM. Its microphone is already wired. Do these steps in order. If one fails, stop and tell me what is wrong and what I should do: 1. Find the new board's serial (COM) port with: pio device list (the PlatformIO tool is at %USERPROFILE%\.platformio\penv\Scripts\pio.exe). Use the port whose hardware ID contains 1A86:55D3, 10C4:EA60 or 303A. Ignore Bluetooth ports. If there is not exactly one such port, stop and ask me. 2. Without showing the password, check that include\wifi_credentials.h exists and no longer contains the text PUT_MY_. If it does, stop and ask me. Tell me which Wi-Fi name the file contains so I can confirm it is the same network as ESP-1. 3. Put the program on the board: pio run -e node -t upload --upload-port <port> 4. Before naming the board, ask it about itself: .venv\Scripts\python.exe tools\provision.py --port <port> --show If it says provisioned=1 with the id ESP-1 (or any id that is already used), stop and ask me, because two boards must never share a name. 5. Give the board its name and Wi-Fi details: .venv\Scripts\python.exe tools\provision.py --port <port> --id ESP-2 --name "Node 2" It must end with the word PROVISIONED and an IP address that is different from ESP-1's. 6. If the dashboard is not already answering at http://127.0.0.1:8080/api/info, start it in the background: .venv\Scripts\python.exe server\app.py --no-browser 7. Check that ESP-2 is listed at http://127.0.0.1:8080/api/nodes with online=true and mic="alive" (wait up to 20 seconds), and that ESP-1 is still listed if it has power. Then record 1 second from ESP-2 as a test: send POST http://127.0.0.1:8080/api/capture with {"node_id":"ESP-2","seconds":1}, ask /api/jobs/<job_id> every second until it is done, then read stats.result at /api/captures/<capture_id>. The result should be "OK". 8. If step 5 printed an IP address but ESP-2 never shows up in step 7, run tools\udp_probe.py for 20 seconds and find out why. Because ESP-1 already worked, the firewall is probably fine, so first look at this board's signal strength, its name and its Wi-Fi details. Explain what you found and ask me before changing anything. Rules: do not change any source file or any GPIO pin number. Use only this project's .venv Python and the PlatformIO tool. Never print the Wi-Fi password. At the end give me a short summary: COM port, IP address, signal strength (rssi), microphone status, and the test recording result. Then ask me to look at the LED on the board: green means good.

## Prompt for ESP-3

Copy everything inside the grey box.

I am a beginner. Please set up AcoustiScan board ESP-3 (display name "Node 3") for me and add it to the dashboard. Explain each step in simple words, one or two lines at a time. Situation: this VS Code window has the AcoustiScan project open. ESP-1 and ESP-2 are already set up (they may be running somewhere else: do not touch them). Exactly one NEW board is plugged into this laptop, in the USB socket marked COM. Its microphone is already wired. Do these steps in order. If one fails, stop and tell me what is wrong and what I should do: 1. Find the new board's serial (COM) port with: pio device list (the PlatformIO tool is at %USERPROFILE%\.platformio\penv\Scripts\pio.exe). Use the port whose hardware ID contains 1A86:55D3, 10C4:EA60 or 303A. Ignore Bluetooth ports. If there is not exactly one such port, stop and ask me. 2. Without showing the password, check that include\wifi_credentials.h exists and no longer contains the text PUT_MY_. If it does, stop and ask me. Tell me which Wi-Fi name the file contains so I can confirm it is the same network as the other boards. 3. Put the program on the board: pio run -e node -t upload --upload-port <port> 4. Before naming the board, ask it about itself: .venv\Scripts\python.exe tools\provision.py --port <port> --show If it says provisioned=1 with the id ESP-1, ESP-2 (or any id that is already used), stop and ask me, because two boards must never share a name. 5. Give the board its name and Wi-Fi details: .venv\Scripts\python.exe tools\provision.py --port <port> --id ESP-3 --name "Node 3" It must end with the word PROVISIONED and an IP address that is different from the other boards'. 6. If the dashboard is not already answering at http://127.0.0.1:8080/api/info, start it in the background: .venv\Scripts\python.exe server\app.py --no-browser 7. Check that ESP-3 is listed at http://127.0.0.1:8080/api/nodes with online=true and mic="alive" (wait up to 20 seconds), and that the earlier boards that have power are still listed. Then record 1 second from ESP-3 as a test: send POST http://127.0.0.1:8080/api/capture with {"node_id":"ESP-3","seconds":1}, ask /api/jobs/<job_id> every second until it is done, then read stats.result at /api/captures/<capture_id>. The result should be "OK". 8. If step 5 printed an IP address but ESP-3 never shows up in step 7, run tools\udp_probe.py for 20 seconds and find out why. Because the earlier boards already worked, the firewall is probably fine, so first look at this board's signal strength, its name and its Wi-Fi details. Explain what you found and ask me before changing anything. Rules: do not change any source file or any GPIO pin number. Use only this project's .venv Python and the PlatformIO tool. Never print the Wi-Fi password. At the end give me a short summary: COM port, IP address, signal strength (rssi), microphone status, and the test recording result. Then ask me to look at the LED on the board: green means good.

## Prompt for ESP-4 (last board, also checks all four)

Copy everything inside the grey box.

I am a beginner. Please set up AcoustiScan board ESP-4 (display name "Node 4") for me and add it to the dashboard. This is the last board, so at the end please check all four boards together. Explain each step in simple words, one or two lines at a time. Situation: this VS Code window has the AcoustiScan project open. ESP-1, ESP-2 and ESP-3 are already set up (they may be running somewhere else: do not touch them). Exactly one NEW board is plugged into this laptop, in the USB socket marked COM. Its microphone is already wired. Do these steps in order. If one fails, stop and tell me what is wrong and what I should do: 1. Find the new board's serial (COM) port with: pio device list (the PlatformIO tool is at %USERPROFILE%\.platformio\penv\Scripts\pio.exe). Use the port whose hardware ID contains 1A86:55D3, 10C4:EA60 or 303A. Ignore Bluetooth ports. If there is not exactly one such port, stop and ask me. 2. Without showing the password, check that include\wifi_credentials.h exists and no longer contains the text PUT_MY_. If it does, stop and ask me. Tell me which Wi-Fi name the file contains so I can confirm it is the same network as the other boards. 3. Put the program on the board: pio run -e node -t upload --upload-port <port> 4. Before naming the board, ask it about itself: .venv\Scripts\python.exe tools\provision.py --port <port> --show If it says provisioned=1 with the id ESP-1, ESP-2, ESP-3 (or any id that is already used), stop and ask me, because two boards must never share a name. 5. Give the board its name and Wi-Fi details: .venv\Scripts\python.exe tools\provision.py --port <port> --id ESP-4 --name "Node 4" It must end with the word PROVISIONED and an IP address that is different from the other boards'. 6. If the dashboard is not already answering at http://127.0.0.1:8080/api/info, start it in the background: .venv\Scripts\python.exe server\app.py --no-browser 7. Check that ESP-4 is listed at http://127.0.0.1:8080/api/nodes with online=true and mic="alive" (wait up to 20 seconds). Then record 1 second from ESP-4 as a test: send POST http://127.0.0.1:8080/api/capture with {"node_id":"ESP-4","seconds":1}, ask /api/jobs/<job_id> every second until it is done, then read stats.result at /api/captures/<capture_id>. The result should be "OK". 8. Final check of all four boards: read /api/nodes and show me a simple table with, for ESP-1 to ESP-4: online (yes/no), IP address (all four must be different), signal strength (rssi) and microphone status. A board without power counts as offline, so tell me which boards still need power. Then record 1 second from each online board, one after another (not at the same time), and add each stats.result to the table. 9. If a board got an IP address in its own setup but never shows up in the dashboard, run tools\udp_probe.py for 20 seconds and find out why (Windows firewall for python.exe on Public networks, client isolation on the hotspot or router, or the laptop being on a different Wi-Fi network). Explain what you found and ask me before changing any firewall setting. Rules: do not change any source file or any GPIO pin number. Use only this project's .venv Python and the PlatformIO tool. Never print the Wi-Fi password. At the end give me a short summary: COM port, IP address, signal strength (rssi), microphone status and test recording result for ESP-4, then the table from step 8. Then ask me to look at the LED on the board: green means good.

# Part 7. Use the dashboard

- Connect the laptop to the same Wi-Fi as the boards (the hotspot must be on).

- Power the boards with chargers or power banks. Wait 10 to 20 seconds for the green LED.

- In the project folder, double-click run_dashboard.cmd. A black window opens and then your browser opens the dashboard at http://127.0.0.1:8080. Do not close the black window. Closing it stops the dashboard.

- In the left column, Nodes, ESP-1 to ESP-4 appear within a few seconds.

- Click a board. Choose 1, 5, 10 or 30 seconds. Optionally type a label, for example "clap test". Click Start capture.

- Wait for the progress bar. The page then shows numbers, the waveform, the spectrum and the spectrogram.

The dashboard records one board at a time. Every recording is saved as a sound file in the folder captures\ESP-1\ (one folder per board), with a small data file next to it. Any audio program can open the sound files.

## What the LED colours mean

| Colour | Meaning |
| --- | --- |
| Blue | Connecting to Wi-Fi. |
| Green | Online and healthy. |
| Red | The microphone is not delivering data. Check the five wires. |
| Yellow | Online, but never named (provisioned). |
| White, blinking | Recording. |
| Cyan | Sending the recording to the laptop. |

# Part 8. When something goes wrong

Look for your problem in the table. If it does not help, use one of the three prompts below.

| Problem | What to do |
| --- | --- |
| The laptop does not see the board | Use another USB cable (it must carry data). Use the socket printed COM. Try another USB port on the laptop. |
| The board is in the dashboard list for a moment, then it is gone | Weak Wi-Fi or two boards with the same name. Move the board closer to the phone. Ask the AI to run the health report (Prompt C below). |
| The board has a Wi-Fi address but is not in the dashboard | Almost always the Windows firewall or client isolation. Use Prompt A below. Quick workaround: in the dashboard, type the board's IP address in Add a node by IP address and click Add. |
| You clicked Cancel on the firewall window | Open PowerShell as Administrator and run the command in the box under this table. Then restart the dashboard. |
| LED stays blue | The board cannot join the Wi-Fi: wrong name or password, hotspot off, or 5 GHz. Check the hotspot band is 2.4 GHz. If the details were wrong, fix include\wifi_credentials.h and use Prompt B. |
| LED is red or the result says "Mic flat" | Check the five wires of the microphone (Part 5). Pin 4 is CLK, pin 5 is DATA, SEL goes to GND. One microphone unit was dead once, so swap it with another if the wires are right. |
| "Python 3.14 not found" | Install Python 3.14 (Part 2) and open a new window. |
| The phone was used and everything stopped | The hotspot probably switched off. Switch it on. The boards reconnect by themselves. |
| You want to change the Wi-Fi | Use Prompt B below. |

New-NetFirewallRule -DisplayName "AcoustiScan discovery (UDP 5005)" -Direction Inbound -Protocol UDP -LocalPort 5005 -Action Allow -Profile Any

## Prompt A: a board does not show up in the dashboard

Replace the number in the first lines with the board that is missing.

I am a beginner. One of my AcoustiScan boards does not show up in the dashboard. The board is ESP-__ (I will tell you the number). Please find out why. Explain in simple words, one or two lines at a time, and stop at the first thing that is wrong. 1. Is the dashboard running? Check http://127.0.0.1:8080/api/info. If not, start it in the background: .venv\Scripts\python.exe server\app.py --no-browser 2. Is this laptop connected to the same Wi-Fi network as the board? Tell me which network the laptop is on. 3. Does the board have a Wi-Fi address? If the board is plugged into this laptop, find its COM port with pio device list (%USERPROFILE%\.platformio\penv\Scripts\pio.exe), then read its messages with pio device monitor -p <port> -b 115200 for about 15 seconds only (use a time limit, then stop it) and look for a line starting with WIFI ip=. Reasons printed there: 201 = network not found (hotspot off, wrong name, or 5 GHz), 202 or 15 or 204 = wrong password. 4. Run tools\udp_probe.py for 20 seconds and tell me whether any board sends a heartbeat. 5. If the board has an IP address but no heartbeat arrives, the usual causes are the Windows firewall (python.exe must be allowed on Public networks, UDP port 5005) or "client isolation" on the hotspot or router. Explain the fix to me in simple words and ask before changing anything. 6. As a workaround, add the board by its IP address: send POST http://127.0.0.1:8080/api/nodes/add with {"ip":"<the board IP>"}. Rules: do not change any source file, GPIO pin number or firewall setting without asking me first. Never print the Wi-Fi password.

## Prompt B: move the boards to another Wi-Fi

First type the new Wi-Fi name and password into include\wifi_credentials.h and save it (Part 4). Then plug in one board and paste this. Repeat for every board.

I am a beginner. I want to move my AcoustiScan boards to a different Wi-Fi network. I have already typed the new Wi-Fi name and password into include\wifi_credentials.h myself. Do NOT show or print the password. Explain in simple words, one or two lines at a time. I will plug in exactly one board at a time, in the USB socket marked COM. For each board: 1. Find its COM port with pio device list (%USERPROFILE%\.platformio\penv\Scripts\pio.exe). Use the port whose hardware ID contains 1A86:55D3, 10C4:EA60 or 303A. If there is not exactly one such port, stop and ask me. 2. Run .venv\Scripts\python.exe tools\provision.py --port <port> --show and tell me which board this is (its id and name). 3. Run .venv\Scripts\python.exe tools\provision.py --port <port> with no --id and no --name. The board keeps its name and takes the Wi-Fi details from include\wifi_credentials.h. It must end with PROVISIONED and an IP address. Tell me the IP address. 4. Ask me to plug in the next board, or to say "done". When I say "done", start the dashboard if it is not running (.venv\Scripts\python.exe server\app.py --no-browser), wait 20 seconds and show me which of ESP-1 to ESP-4 are online. Rules: do not change any source file or any GPIO pin number. Never print the Wi-Fi password.

## Prompt C: health report of all boards

Use it whenever you want to know which boards are online and healthy.

I am a beginner. Please give me a health report of all my AcoustiScan boards. Explain in simple words. 1. Make sure the dashboard is running at http://127.0.0.1:8080/api/info. If not, start it in the background: .venv\Scripts\python.exe server\app.py --no-browser 2. Wait 15 seconds, then read http://127.0.0.1:8080/api/nodes. 3. Show me a simple table with one row per board: id, name, online (yes/no), IP address, signal strength (rssi), microphone (alive or flat) and state. 4. For every board that is online, record 1 second, one board after another: send POST http://127.0.0.1:8080/api/capture with {"node_id":"<id>","seconds":1}, ask /api/jobs/<job_id> every second until it is done, then read stats.result at /api/captures/<capture_id>. Add the result (OK, Mic flat, ...) to the table. 5. Tell me which of ESP-1, ESP-2, ESP-3 and ESP-4 are missing, and what to check first for each missing one. Also explain what a signal weaker than -75 dBm means for me. Rules: do not change any source file, GPIO pin number or setting.

# Part 9. Checklist and the same steps without AI

## Checklist

| Board | Name in dashboard | Mic wired | Label on board | Program + name done | Seen in dashboard | Test recording OK |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | ESP-1 / Node 1 | ☐ | ☐ | ☐ | ☐ | ☐ |
| 2 | ESP-2 / Node 2 | ☐ | ☐ | ☐ | ☐ | ☐ |
| 3 | ESP-3 / Node 3 | ☐ | ☐ | ☐ | ☐ | ☐ |
| 4 | ESP-4 / Node 4 | ☐ | ☐ | ☐ | ☐ | ☐ |

## The same steps by hand (if you have no AI)

Do this for one board at a time, in PowerShell, in the project folder (in VS Code: Terminal > New Terminal). Replace COM8 with your port and ESP-1 / Node 1 with the board's name.

$pio = "$env:USERPROFILE\.platformio\penv\Scripts\pio.exe" $py = ".\.venv\Scripts\python.exe" # 1. find the port (look for hardware ID 1A86:55D3, 10C4:EA60 or 303A) & $pio device list # 2. put the program on the board & $pio run -e node -t upload --upload-port COM8 # 3. give the board its name and Wi-Fi details; it must end with PROVISIONED & $py tools\provision.py --port COM8 --id ESP-1 --name "Node 1" # 4. start the dashboard (or double-click run_dashboard.cmd) & $py server\app.py

More technical details are in the files TEAM_GUIDE.md, README.md and PROTOCOL.md in the project folder.